| SOP ID | PEC-SAFETY-001 |
| --- | --- |
| Title | Jobsite PPE and Safety Requirements -- Epoxy and Concrete Grinding |
| Company | Prescott Epoxy Company (PEC) |
| Department | SAFETY |
| Version | 1.0 |
| Effective Date | April 11, 2026 |
| Owner | Dylan Nordby |

# Purpose

This SOP establishes mandatory personal protective equipment (PPE) requirements and safety procedures for all PEC crew members working with epoxy coatings, polyaspartic topcoats, and concrete grinding equipment. The goal is to prevent injuries, chemical exposure, hearing damage, and respiratory harm on every job.

# Scope

Applies to all PEC employees and subcontractors who are present on a jobsite during any phase of a garage floor coating project, including:

- Concrete surface preparation (grinding, shot blasting, patching)

- Epoxy base coat and flake application

- Polyaspartic topcoat application

- Cleanup and equipment breakdown

This SOP is effective from the moment you arrive on site until all tools are loaded and you leave the property.

# Procedure: Required PPE by Task

## 1. Respiratory Protection

Epoxy resins, hardeners, and polyaspartic coatings release volatile organic compounds (VOCs) and isocyanates. Concrete grinding generates silica dust. Both are serious health hazards with long-term consequences.

Required equipment:

- Half-face respirator with organic vapor/P100 combination cartridges (3M 6000 series or equivalent)

- Cartridges must be replaced at least every 40 hours of use or when you smell breakthrough (whichever comes first)

- During concrete grinding: P100 particulate filters minimum; OV/P100 combo cartridges preferred

When to wear:

- Respirator goes on BEFORE opening any epoxy, hardener, or topcoat container

- Stays on during all mixing, application, and flake broadcast

- Stays on during all grinding and dust-generating prep work

- Do not remove until you are outside the garage with adequate fresh air

Fit check:

- Perform a negative pressure seal check every time you put the respirator on: cover cartridge inlets with palms, inhale sharply. Mask should pull tight to face with no air leaks

- Facial hair that breaks the seal means the respirator does not work. Clean-shaven where the seal sits, no exceptions

## 2. Hearing Protection

Concrete grinders produce sustained noise levels above 90 dB, which causes permanent hearing damage over time.

Required equipment:

- Foam earplugs (NRR 32 minimum) OR over-ear earmuffs (NRR 25 minimum)

- Both may be worn together during extended grinding sessions for additional protection

When to wear:

- During all concrete grinding, shot blasting, and any power tool operation

- If you have to raise your voice to talk to someone 3 feet away, hearing protection is required

## 3. Eye Protection

Chemical splashes, grinding debris, and flake particles all pose direct eye hazards.

Required equipment:

- Safety glasses with side shields (ANSI Z87.1 rated) at all times on site

- Chemical splash goggles during mixing and pouring of epoxy/hardener/topcoat

- Full-face shield over safety glasses during grinding

When to wear:

- Safety glasses: the moment you step on the jobsite until you leave

- Splash goggles: any time you are opening, mixing, or pouring chemicals

- Face shield: all grinding and surface prep operations

## 4. Hand and Skin Protection

Epoxy resins and hardeners cause chemical burns and skin sensitization. Once you develop an allergy to epoxy, it is permanent and career-ending.

Required equipment:

- Nitrile gloves (minimum 6 mil thickness) during all chemical handling -- latex is NOT acceptable (does not block epoxy)

- Change gloves immediately if torn, punctured, or visibly contaminated

- Heavy-duty leather or mechanics gloves during grinding and equipment handling

Skin coverage:

- Long sleeves recommended during application phases. Bare forearms contacting epoxy will cause irritation or burns

- If epoxy contacts skin: wipe off immediately with a dry rag, then wash with soap and water. Do NOT use solvents (acetone, xylene) on skin

## 5. Clothing and Footwear

Required:

- Closed-toe work boots with non-slip soles (steel toe recommended but not required)

- Long pants -- no shorts on the jobsite

- Company shirt or high-visibility shirt

- Shoe covers or dedicated work shoes to avoid tracking material into customer's home

Not allowed:

- Open-toed shoes, sandals, or sneakers

- Loose clothing or jewelry that could catch in grinders

- Headphones or earbuds (hearing protection only)

# Quick Reference: PPE by Task

| Task | Respirator | Hearing | Eyes | Gloves | Notes |
| --- | --- | --- | --- | --- | --- |
| Concrete Grinding | OV/P100 | Required | Shield + glasses | Leather/mech | Dust control on |
| Mixing Epoxy | OV/P100 | -- | Splash goggles | Nitrile 6mil | Ventilate area |
| Epoxy Application | OV/P100 | -- | Safety glasses | Nitrile 6mil | Long sleeves |
| Flake Broadcast | OV/P100 | -- | Safety glasses | Nitrile 6mil | -- |
| Topcoat Application | OV/P100 | -- | Splash goggles | Nitrile 6mil | Fast cure = more vapor |
| Cleanup | If chemicals open | -- | Safety glasses | Nitrile 6mil | Dispose gloves properly |

# Tools and Materials

Each crew truck must carry the following PPE inventory at all times:

- Half-face respirators (one per crew member, properly fitted)

- OV/P100 combination cartridges (minimum 2 spare sets per truck)

- Foam earplugs (box of 50 pairs minimum)

- Over-ear earmuffs (1 set per crew member)

- ANSI Z87.1 safety glasses (3 spare pairs per truck)

- Chemical splash goggles (1 per crew member)

- Face shields (1 per crew member doing grinding)

- Nitrile gloves, 6 mil (box of 100, restocked weekly)

- Leather or mechanics gloves (1 pair per crew member)

- First aid kit (OSHA compliant, checked monthly)

- Eyewash station or bottles (minimum 2 per truck)

Crew leads are responsible for checking PPE inventory before leaving the shop each morning. If any item is missing or depleted, notify the Office Manager (Dusty) before heading to the job.

# Common Mistakes

1. Taking the respirator off too early.

VOCs continue off-gassing after application. Keep your respirator on until you are outside the work area with fresh air circulation. The polyaspartic topcoat is especially bad for this because it cures fast and people assume that means the vapor is gone. It is not.

2. Using latex gloves instead of nitrile.

Latex does not block epoxy resin. It will soak through within minutes. You will not feel it happening until the burn starts. Nitrile only, 6 mil minimum. If someone shows up with latex, they do not work with chemicals until they have the right gloves.

3. Skipping hearing protection during "quick" grinding.

There is no safe duration for unprotected exposure above 90 dB. Even 5 minutes of grinding without ear protection causes cumulative damage. Every time, no exceptions.

4. Not performing respirator seal checks.

A respirator that does not seal is just a face decoration. Negative pressure check every time you put it on. If you have stubble or a beard where the seal sits, it is not sealing. Shave or use a PAPR system.

5. Using solvents to clean epoxy off skin.

Acetone and xylene strip the protective oils from your skin and can drive chemicals deeper. Wipe with a dry rag first, then soap and water. That is it.

# Emergency Procedures

## Chemical Splash to Eyes

- Immediately flush eyes with clean water or eyewash for a minimum of 15 minutes

- Remove contact lenses if present (do not delay flushing to do this)

- Have someone call 911 if irritation continues or vision is affected

- Crew lead reports incident to Dylan Nordby immediately

## Chemical Skin Contact

- Remove contaminated clothing immediately

- Wipe excess material off with a dry rag (not a solvent)

- Wash area thoroughly with soap and water for 5 minutes

- If redness, burning, or blistering develops, seek medical attention

## Respiratory Distress

- Move the person to fresh air immediately

- If difficulty breathing continues, call 911

- Do not re-enter the work area until ventilation is confirmed adequate

## Injury Reporting

All injuries, no matter how minor, must be reported to the crew lead on the same day. The crew lead reports to Dylan Nordby and Dusty (Office Manager) within 24 hours. See the Employee Handbook Section 5.5 for Workers' Compensation procedures.

# Revision History

| Date | Author | Changes |
| --- | --- | --- |
| 2026-04-07 | Dylan Nordby | Initial version. Covers PPE requirements for epoxy coating and concrete grinding operations. |