# Off-Site Procedures and Truck Inventory

| SOP ID | PEC-OPS-011 |
| --- | --- |
| Company | Prescott Epoxy Company (PEC) |
| Department | OPS |
| Version | 1.0 |
| Effective Date | 2026-04-13 |
| Owner | Dylan Nordby |

## Purpose

Standardize truck stocking, equipment maintenance, and vehicle care procedures to ensure crews are always prepared for the next job.

## Scope

All PEC crew leads responsible for truck inventory and off-site maintenance.

## Tools and Materials

- Truck inventory checklist

- Propane cylinders

- Diamond inventory

- Consumable supplies (rollers, brushes, cups, gloves, etc.)

- Pressure washer (for truck cleaning)

## Procedure

### Consumable Supplies Inventory

- Rollers: We typically go through 3-4 per day. Keep 20 rollers on the truck each week. Notify office when running out.

- Brushes: Keep 40 brushes per week (minimum 2 per job per person on the floor).

- Patching materials: Carry a kit of fast cure epoxy and thickening fibers, one bag of Henrys, and a kit of polymer-based patch.

- Trash bags: Carry one box of contractor grade bags. Notify office when halfway through.

- Gloves: Carry 2 boxes on all trucks. Use around 3 pairs per person per coating. Notify office when first box is empty.

- Respirator filters: Change weekly. Concrete dust restricts airflow.

- Rags: Carry one large bag of terry towels (2 rolls). Notify office when through the first roll.

- Cups: Carry 15 cups per day on each truck for patching, measuring, and edge work.

- Longo packs: Carry one box. Notify office when down to the last bag.

### Equipment Inventory (Per Truck)

- One industrial grinder

- One 7-inch hand grinder and one 4-inch grinder with shrouds for both

- Two vacuums (for hand units and industrial unit)

- 4 poles: one with 18-inch frame, one with notched squeegee, one with flat squeegee, one with 6-inch weenie roller frame

- Broom and dust pan

- Microfiber mop for scrub and seals and acetone wipes

- 5 scrub and seal microfiber pads and 5 acetone microfiber pads

- Trash can with wheels for moving heavy loads

### Diamond Inventory (Per Truck)

Need one of each bond (super soft, soft, medium, hard) in 16/18, 30/40, and 60/80 for both industrial and hand grinders.

Once diamonds are ground down halfway, notify the office for reorder.

### Diamond Wear Check Schedule

Leads should go through diamond and tool inventory weekly.

Every Monday and Friday, check diamond wear and note what you have and do not have. If dwindling, bring that information to the office for ordering.

### Propane and Gas Procedures

- Every Monday, grab new propane cylinders from the shop and leave empties in the drop-off area.

- For gas (truck and generators), fill up after the job on Fridays to set up for the next week.

- If low on fuel mid-week, notify the office.

### Keeping the Truck Clean

- While the lead fills gas, another installer cleans out trash and keeps the dash and floor clear.

- If eating in the truck, keep trash in a bag or lunch box.

- The lead in charge of the truck brings it to the shop on a weekend and pressure washes it on the clock.

### Vehicle Maintenance

- Track mileage for oil changes.

- Monitor tire wear, mileage, and year of manufacture.

- Notify Dylan of maintenance needed and make calls for service per Dylan's budget and plans.

### Vehicle Damage Reporting

Report the following to Dylan/insurance as soon as possible: tire problems, steering problems, windshield cracks, lift gate issues, starting problems, and crashes/vehicle disability.

### Equipment Maintenance

- Vacuums: Clear HEPA filters of debris for maximum dust control. Prevents overworking.

- Generators: Show a code or stop running if oil is not changed every 100 hours.

- Propane grinders: Need an oil and oil filter change every 50 grinding hours.

## Revision History

| Date | Author | Changes |
| --- | --- | --- |
| 2026-04-13 | Dylan Nordby / SOP Manager | Initial version created from training documentation. |

Source: SOP Hub | Generated 2026-04-13